Energy Solutions

RR 5  Shawville  Québec J0X 2Y0

Voice 819.647.5092

Fax  819.647.6082

email mheat@hookup.net

# Construction Drawings—Cookstove with right side exit

| Top View | Isometric View |
| --- | --- |
| Front View | Front Section Not shown is grate, which is 8 ¼ × 16 1/8" and sits on ledge in doorset. |
| Outer masonry is shown as randomly coursed 4” stone - adjust as necessary for brick | Lay bond break (tarpaper or plastic) on floor. Lay down full bedjoint with a piece of diamond lath embedded in it. Fill centre section with 1:4 portland:vermiculite mix. |
| Corbel out over kickspace as shown. Fill approximate centre section under bakeoven with 1:4 portland:vermiculite mix. |  |
| Set 2 ½" firebrick pad as shown. Dry cross and head joints are OK. Start oven floor support as shown using firebrick soaps (ripped lengthwise) Joints on firebrick pad not shown. |  |
| Start support for channel ceiling. Shown as 2x2 and 1x2 angle iron. Set flue liner connection from oven bottom. Notch as shown to line up with exit channel. | Angle iron detail |
| Lay in ceiling as shown. Mortar firebricks with firecement | Make flue cutout to match cookstove exit underneath bakeoven. Front angle iron shown shorter than actual (refer to previous frame) |
|  | Leave gap for gas channel as shown. |
| Bring up facing. Install 8x8 cleanout as shown. Leave opening in right side facing for bypass connection. Notch horizontal connecting flue for bypass. Check bypass installation (bypass may require some modification). Line with splits as shown. Not shown, but splits should actually return around corners and butt up against bakeoven. Set bakeoven up on 2 ½" supports, not shown. | Set oven in place. Use thin strip of ceramic blanket against facing. Bring up firebox as shown. Missing from these plans is the grate. It is 8 ¼ x 16 1/8. Notch firebrick at correct height (determined by location of ledge for grate on door). Note splits at rear to gain space. Note mitered corner on left. Bond in on right side where possible. Set firebricks tight against oven. Shown without facing for clarity, but can be brought up after facing or at same time as facing. |
| Section through cookstove. |  |
| Note exposed firebrick strip on left side of oven. This is a weak area for brick coostove design. Facing brick can be used here, but should be bonde somehow. Siliconing directly to bakeoven is probably the best. | Lay in two cap strips as shown. Strongest is sand mix with mesh. Could use strips of stone also, preferably with some reinforcement. Mount coostove door with tapcons similar to heater doors, using ceramic blanket for expansion. Top of door frame should hook underneath cooktop. Set cooktop in place with ¾" gastket rope or equivalent. |